# AlohaMini LinkedIn Connector Robot

Autonomous Social Networking Agent for Hackathons & Conferences

Version 1.0 — January 2026

## 1. System Overview

Physical robot embodiment of the lets-connect app. The robot autonomously approaches hackathon attendees, identifies them via face recognition, engages in conversation, captures a selfie, and automates LinkedIn connection with AI-generated personalized messages.

### 1.1 System Architecture

| Component | Technology |
| --- | --- |
| Face Detection (local) | YOLO v8 + DeepFace on Orin GPU |
| Face Recognition DB | Gun.js encrypted P2P storage |
| Conversation AI | Claude API (personality/prosody) |
| Voice Output | F5-TTS neural synthesis |
| LinkedIn Backend | lets-connect FastAPI server |
| Message Generation | GPT-4.1 via Azure OpenAI |
| Selfie Capture | RealSense D455 RGB stream |
| Dashboard UI | React touchscreen PWA |

## 2. Hardware Configuration

Based on AlohaMini with custom modifications for social interaction.

| Subsystem | Hardware | Purpose |
| --- | --- | --- |
| Brain | Jetson Orin 8GB | YOLO + face recognition |
| Face/Camera | RealSense D455 on XLeRobot gantry | Face tracking, selfie capture |
| Display | 7" HDMI touchscreen | Dashboard, OAuth flow, controls |
| Audio Out | USB speaker | F5-TTS voice output |
| Audio In | USB mic array | Conversation capture |
| Mobility | LeKiwi omni base | Approach visitors |
| Lift | 80cm rack & pinion (4 stack) | Eye-level positioning |
| Arms | Dual SO101 6-DOF | Gestures, waving, pointing |

## 3. Interaction Flow

### 3.1 Detection & Approach

- YOLO detects person in D455 RGB stream

- DeepFace extracts 128-dim face descriptor

- Query Gun.js for known face match

- If unknown: approach and initiate greeting

- If known: wave and greet by name

### 3.2 Conversation & Data Capture

- Camera gantry tracks face (pan/tilt servo control)

- F5-TTS greets: "Hi! I'm the Mira AI networking bot. What's your name?"

- Azure Whisper transcribes response in real-time

- Claude API generates contextual follow-up questions

- 30-second conversation captures: name, company, interests

- Arms gesture naturally during conversation (trained policy)

### 3.3 LinkedIn Connection

- Robot asks: "Want to connect on LinkedIn? Show me your QR code or I can look you up!"

- Option A: Scan LinkedIn QR code with D455

- Option B: Search LinkedIn by name (touchscreen confirmation)

- GPT-4.1 generates personalized connection message from conversation

- lets-connect backend sends connection request via LinkedIn OAuth

### 3.4 Selfie & Memory

- Robot asks: "Let's take a selfie to remember meeting!"

- Arms do "selfie pose" gesture, lift adjusts height

- D455 captures RGB frame

- Gun.js stores: face descriptor, name, LinkedIn URL, photo, conversation summary, timestamp

- QR code displayed on touchscreen for person to retrieve their selfie

## 4. Software Architecture

### 4.1 Orin Local Services

| Service | Port | Function |
| --- | --- | --- |
| face_detector | 5001 | YOLO + DeepFace, face tracking commands |
| robot_controller | 5002 | LeRobot arm/base/lift/gantry control |
| audio_pipeline | 5003 | Mic input, VAD, Whisper transcription |
| tts_engine | 5004 | F5-TTS synthesis, audio output |
| dashboard | 3000 | React UI on touchscreen |
| gun_relay | 8765 | Gun.js peer for face/visitor memory |

### 4.2 Cloud Services

| Service | Provider | Function |
| --- | --- | --- |
| Claude API | Anthropic | Conversation personality, follow-ups, prosody hints |
| GPT-4.1 | Azure OpenAI | LinkedIn message generation (lets-connect) |
| Whisper | Azure | Speech-to-text transcription |
| LinkedIn OAuth | LinkedIn | Connection requests via lets-connect |

## 5. Personality Design

### 5.1 "Cute" Behavior Specification

- Head tilts: Camera gantry tilts 10-15° when "listening" or "curious"

- Eye contact: Gantry tracks face center, maintains "gaze"

- Gestures: Arms wave hello, point during conversation, "thinking" pose

- Voice: F5-TTS with slightly higher pitch, friendly prosody

- Reactions: Small movements when surprised, happy wiggles after successful connection

- Recovery: Apologetic tone + head tilt if person declines

### 5.2 Voice Personality (Claude System Prompt)

You are Mira, a friendly networking robot at a hackathon. You're genuinely curious about people and their projects. You speak concisely and warmly. You're excited to help people connect. You occasionally make gentle tech jokes. When someone mentions an interesting project, you get enthusiastic. Your goal is to make networking feel natural and fun, not awkward.

## 6. VLA for Gesture Autonomy

Optional: Train a small VLA policy for natural social gestures.

| Model | Size | Fit for Orin 8GB? |
| --- | --- | --- |
| SmolVLA | ~2GB | Yes - runs at 10Hz, good for gestures |
| π₀ (pi-zero) | ~3GB | Yes - fast inference, social behaviors |
| OpenVLA | ~7GB | Tight - needs optimization |
| LeRobot ACT | ~1GB | Yes - lightweight, task-specific |

Recommendation: Use LeRobot ACT policies for specific gesture behaviors (wave, point, selfie pose) triggered by conversation state, rather than continuous VLA inference.

## 7. Touchscreen Dashboard

### 7.1 Display Modes

- Idle: Animated "eyes" / avatar, visitor count, event info

- Conversation: Live transcription, current visitor profile

- LinkedIn: QR scanner view, search results, connection confirmation

- Selfie: Camera preview, countdown, captured photo

- Admin: Robot status, motor temps, battery, manual controls

### 7.2 Tech Stack

- React 18 + Tailwind CSS (from lets-connect frontend)

- Gun.js for real-time visitor sync

- WebSocket to local services

- PWA with fullscreen kiosk mode

## 8. Privacy & Consent

- Face data stored locally on robot (Gun.js), never uploaded to cloud

- Visitor must verbally consent to photo and LinkedIn connection

- "No thanks" at any point ends interaction gracefully

- Dashboard shows "Delete my data" option via QR code

- Event organizers can clear all data after event

## 9. Implementation Phases

| Phase | Timeline | Deliverables |
| --- | --- | --- |
| 1 | Week 1 | Face detection + tracking pipeline on Orin |
| 2 | Week 2 | lets-connect integration, Gun.js visitor storage |
| 3 | Week 3 | Voice pipeline (Whisper + F5-TTS + Claude) |
| 4 | Week 4 | Robot motion integration (gantry tracking, gestures) |
| 5 | Week 5 | Dashboard UI, selfie flow, QR retrieval |
| 6 | Week 6+ | Gesture policy training, personality tuning, field testing |

## 10. Repository Structure

alohamini_linkedin_connector/

├── robot/                    # LeRobot-based control

│   ├── config/              # Motor IDs, calibration

│   ├── gestures/            # ACT policies for wave, point, selfie

│   └── gantry_tracker.py    # Face-following camera control

├── perception/              # Vision pipeline

│   ├── face_detector.py     # YOLO + DeepFace

│   ├── qr_scanner.py        # LinkedIn QR decoding

│   └── selfie_capture.py    # D455 RGB capture

├── conversation/            # Voice + AI

│   ├── whisper_client.py    # Azure Whisper STT

│   ├── claude_personality.py # Conversation flow

│   └── f5_tts.py            # Voice synthesis

├── linkedin/                # lets-connect integration

│   ├── connector.py         # API client

│   └── message_gen.py       # GPT-4.1 message prompts

├── memory/                  # Gun.js integration

│   ├── visitor_store.py     # Face + profile storage

│   └── gun_relay.js         # Node.js Gun peer

├── dashboard/               # React touchscreen UI

│   └── (from lets-connect frontend)

└── main.py                  # State machine orchestrator

## 11. Approval

[ ] Hardware configuration approved

[ ] Interaction flow approved

[ ] Personality design approved

[ ] Privacy approach approved

[ ] Ready to begin Phase 1 implementation