# AlohaMini-Orin: Vertical Stack Mobile Manipulator

Planning Document v1.0 | January 30, 2026

Status: AWAITING APPROVAL

## 1. Executive Summary

Complete rebuild of AlohaMini platform with Jetson Orin 8GB brain, vertical rack-and-pinion lift, dual SO101 arms on independent gantries, and elevated camera gantry for multi-level workspace coverage (floor to above-counter).

## 2. Hardware Stack

| Layer | Component | Specs |
| --- | --- | --- |
| Brain | Jetson Orin 8GB | 1024 CUDA, 8-core ARM |
| Power | Anker Solix | 300W, USB-C PD |
| Vision | Intel RealSense D455 | RGBD, 87° FOV, USB3 |
| Base | AlohaMini omni-drive | 3x Feetech STS3215 (IDs 7,8,9) |
| Lift | Rack & pinion vertical | 0-60cm travel, motor TBD |
| Arms (x2) | SO101 6-DOF each | 12x Feetech STS3215 total |
| Arm Gantries | Independent per arm | Pan axis per arm |
| Camera Gantry | 2-DOF (pan/tilt) | Above-counter height |

## 3. Mechanical Architecture

### 3.1 Vertical Stack (Bottom to Top)

- Omni-wheel base (3-wheel, 120° spacing)

- Battery + Jetson Orin compartment

- Rack & pinion lift stage (vertical travel)

- Dual arm mounting plate with independent gantries

- SO101 Left Arm (6-DOF)

- SO101 Right Arm (6-DOF)

- Camera hat gantry (2-DOF pan/tilt)

- RealSense D455 sensor head

### 3.2 Workspace Zones

| Zone | Height Range | Tasks | Lift Position |
| --- | --- | --- | --- |
| Floor | 0-20cm | Pickup, vacuuming | Fully down |
| Table | 70-90cm | Dishes, objects | Mid |
| Counter | 90-110cm | Kitchen tasks | High |
| Above-counter | 110-140cm | Shelves, cabinets | Full up + camera |

## 4. Motor ID Assignment

| ID | Joint | Bus | Notes |
| --- | --- | --- | --- |
| 1-6 | Left Arm (base→gripper) | Bus A (/dev/ttyACM0) | SO101 standard |
| 11-16 | Right Arm (base→gripper) | Bus B (/dev/ttyACM1) | SO101 standard |
| 7,8,9 | Wheels (LF, RF, Rear) | Bus A | LeKiwi standard |
| 20 | Lift motor | Bus A | Rack & pinion |
| 21 | Left arm gantry pan | Bus A | Arm positioning |
| 22 | Right arm gantry pan | Bus B | Arm positioning |
| 23 | Camera pan | Bus B | Head tracking |
| 24 | Camera tilt | Bus B | Head tracking |

## 5. Software Architecture

### 5.1 Stack Layers

| Layer | Component | Function |
| --- | --- | --- |
| Hardware | Feetech SDK | Motor control, feedback |
| Perception | RealSense SDK + pyrealsense2 | RGBD streaming, depth |
| Robot Control | LeRobot | Arms, base, teleoperation |
| Intelligence | WhoAmI | Face recognition, voice, memory |
| Policy | ACT / Diffusion / SmolVLA | Learned behaviors |

### 5.2 Custom Extensions Required

- alohamini_orin.py - Robot class extending LeKiwi with lift + dual arms

- lift_controller.py - Rack & pinion lift control with position feedback

- dual_arm_controller.py - Coordinated bimanual control

- camera_gantry.py - 2-DOF head tracking with face following

- workspace_manager.py - Zone-based lift presets (floor/table/counter)

- config/alohamini_orin.yaml - Hardware configuration file

### 5.3 WhoAmI Integration Points

- Face detection via D455 RGB → YOLO on Orin GPU

- Face tracking → camera gantry follows detected face

- Name collection → F5-TTS voice interaction

- Gun.js memory → local encrypted person database

- Conversation tracking → recall previous interactions

## 6. Implementation Phases

### Phase 1: Base Platform (Week 1)

- Orin setup with JetPack 6.x

- RealSense D455 driver installation (JetsonHacks method)

- LeRobot installation with Jetson PyTorch wheels

- Motor ID configuration for all axes

- Basic teleoperation test (single arm)

### Phase 2: Extended Kinematics (Week 2)

- Lift controller with homing sequence

- Dual arm calibration and coordination

- Camera gantry tracking implementation

- Workspace zone presets

### Phase 3: Intelligence Layer (Week 3)

- WhoAmI face recognition pipeline

- F5-TTS voice synthesis

- Gun.js person memory

- Face-following behavior

### Phase 4: Task Training (Week 4+)

- Dataset collection for target tasks

- Policy training (ACT or diffusion)

- Evaluation and iteration

## 7. Proposed File Structure

alohamini_orin/
├── config/
│   └── alohamini_orin.yaml       # Hardware config
├── src/
│   ├── robot/
│   │   ├── alohamini_orin.py     # Main robot class
│   │   ├── lift_controller.py    # Vertical lift
│   │   ├── dual_arm_controller.py
│   │   ├── camera_gantry.py
│   │   └── workspace_manager.py
│   ├── perception/
│   │   ├── realsense_interface.py
│   │   └── face_tracker.py
│   └── intelligence/
│       └── whoami_integration.py
├── scripts/
│   ├── setup_motors.sh
│   ├── calibrate.sh
│   └── run_teleop.py
└── README.md

## 8. Approval Checklist

- [ ] Hardware stack accurate

- [ ] Motor ID assignment approved

- [ ] Workspace zones correct

- [ ] Software architecture approved

- [ ] Phase timeline acceptable

- [ ] File structure approved

## 9. Open Questions

- Lift motor model - same STS3215 or different actuator?

- Lift travel distance - confirm 60cm or adjust?

- Arm gantry DOF - pan only or pan+slide?

- Leader arms for teleoperation - one or two?

- Primary tasks for initial training?

Prepared by: Claude | Awaiting approval from 0101 0001