# Johnny Five — Hackathon Build

48 Hours to Magic

January 31 – February 2, 2026

Built by Alan Helmick & Opus the Magnificent

## Mission

Build a robot that networks better than humans. Johnny Five approaches people, recognizes faces, greets by name with millisecond precision, has real conversations, begs for selfies, and connects on LinkedIn — all without needing Premium.

## Already Working

- WhoAmI face recognition on Jetson Orin 8GB + OAK-D

- YOLO v8 overlays baked into OAK-D DepthAI pipeline

- Name prompt → bounding box labels in real-time

- Time-since-last-seen greeting ("X min, X sec, X ms!")

- Gun.js encrypted local face/visitor storage

- lets-connect backend (FastAPI + LinkedIn OAuth + GPT-4.1)

- AlohaMini rebuilt: 80cm lift, XLeRobot gantry, dual SO101 arms

- 7" touchscreen front-mounted

## Build List (Priority Order)

| # | Task | Details | Time |
| --- | --- | --- | --- |
| 1 | Gantry face tracking | PID servo control from OAK-D bbox center | 2 hrs |
| 2 | Saccadic motion | Quick dart → overshoot → settle (Johnny Five feel) | 2 hrs |
| 3 | Idle behaviors | Curious scanning, micro-movements when no face | 1 hr |
| 4 | F5-TTS voice | Greetings, conversation, selfie request | 2 hrs |
| 5 | Selfie capture | OAK-D RGB → QR code for retrieval | 2 hrs |
| 6 | LinkedIn integration | lets-connect: search/QR → AI message → send | 3 hrs |
| 7 | Arm gestures | Wave, point, selfie pose synced to conversation | 3 hrs |
| 8 | Dashboard polish | React UI: POV + YOLO + visitor count + controls | 2 hrs |

## Timeline

Day 1 — Saturday January 31

- 9 AM: WhoAmI running on rebuilt unit

- 12 PM: Gantry tracking + saccadic motion

- 3 PM: F5-TTS voice + idle behaviors

- 6 PM: Selfie flow working

- 9 PM: Demo 1 — Johnny greets, tracks, speaks, captures selfies

Day 2 — Sunday February 1

- 9 AM: lets-connect LinkedIn integration

- 12 PM: "Send me our selfie!" message flow

- 3 PM: Arm gestures (wave, point, selfie pose)

- 6 PM: Full integration test

- 9 PM: Demo 2 — Full flow, live at hackathon

## The Hook

Johnny approaches people at the hackathon. Recognizes faces. Greets by name with exact time since last meeting ("Hi Sarah! 3 minutes, 42 seconds, 891 milliseconds since we last met!"). Has a real conversation powered by Claude. Then Johnny begs for a selfie.

After the selfie, Johnny shows a QR code to retrieve it and sends a LinkedIn connection request with an AI-generated message:

"Great meeting you at the hackathon! Here's our selfie — send it to me so I have it too!"

They message first → no LinkedIn Premium needed. Johnny follows up if they don't send it.

## Hardware Stack

| Component | Spec |
| --- | --- |
| Brain | Jetson Orin 8GB (1024 CUDA, 8-core ARM) |
| Face Camera | OAK-D on XLeRobot gantry (2-DOF pan/tilt, IDs 21-22) |
| Wrist Cams | 2x 180° 1080p on SO101 arms |
| Display | 7" HDMI touchscreen (via DP-to-HDMI adapter) |
| Arms | Dual SO101 6-DOF (Left 1-6, Right 11-16) |
| Lift | 80cm rack & pinion (ID 10, 4 stack pieces) |
| Base | LeKiwi omni-drive (IDs 7, 8, 9) |
| Power | Anker Solix 300W (12V/10A = 120W available) |
| Audio | USB speaker (PulseAudio, bypasses ALSA) |

## Power Wiring

Simple direct power from Anker Solix 12V output:

Anker Solix 12V ──┬──> Orin barrel jack (12V direct, within 9-20V spec)

│

└──> Waveshare servo drivers (already wired)

Common ground through barrel returns.

No GPIO header wiring needed. Orin barrel jack accepts 9-20V, Anker delivers 12V/10A.

## Software Stack

| Layer | Tech |
| --- | --- |
| Face Recognition | WhoAmI (YOLO v8 + DeepFace on OAK-D DepthAI) |
| Memory | Gun.js (local encrypted, robot-owned data) |
| Voice Synthesis | F5-TTS neural TTS |
| Conversation | Claude API (personality + prosody hints) |
| LinkedIn | lets-connect (FastAPI + GPT-4.1 messages) |
| Robot Control | LeRobot / Feetech SDK |
| Dashboard | React (from lets-connect frontend) |
| Audio | PulseAudio over USB speaker |

## Johnny Five Personality

Inspired by Short Circuit — the goal is "alive," not "smooth."

- Saccadic gaze: quick dart → overshoot → settle (never smooth pursuit)

- Reaction delay: 100-200ms pause before responding (feels like thinking)

- Head tilts: 10-15° when curious or listening

- Idle scanning: small random movements, curious about environment

- Attention shifts: notice → pause → turn (not instant lock)

- Happy wiggles: after successful selfie or LinkedIn connection

- Beat gestures: arms move with speech rhythm

### Conversation System Prompt

You are Mira, a friendly networking robot at a hackathon. You're genuinely curious about people and their projects. You speak concisely and warmly. You're excited to help people connect. You occasionally make gentle tech jokes. When someone mentions an interesting project, you get enthusiastic. Your goal is to make networking feel natural and fun, not awkward.

## Code Structure

Minimal new code — most functionality comes from existing repos:

johnny5/

├── gantry_tracker.py      # PID face tracking + saccades

├── personality.py         # Idle behaviors, reaction timing

├── gestures.py            # Arm wave, point, selfie pose

├── selfie_flow.py         # Capture + QR generation

├── linkedin_bridge.py     # lets-connect API client

├── voice.py               # F5-TTS + PulseAudio output

└── main.py                # State machine orchestrator

### Gantry Face Tracking

PID control from OAK-D face detection:

# gantry_tracker.py

class SaccadicTracker:

def update(self, face_bbox):

# Get face center

target_x = (bbox.x1 + bbox.x2) / 2

target_y = (bbox.y1 + bbox.y2) / 2

# Saccadic motion: quick move with overshoot

if distance_to_target > threshold:

# Fast saccade

overshoot = 1.15  # 15% overshoot

move_pan(target_x * overshoot)

move_tilt(target_y * overshoot)

sleep(0.05)  # Brief pause

# Settle to actual target

move_pan(target_x)

move_tilt(target_y)

# Add reaction delay (100-200ms)

sleep(random.uniform(0.1, 0.2))

### State Machine

States:

IDLE          → Scanning, micro-movements, waiting for face

FACE_DETECTED → Saccade to face, start recognition

GREETING      → Play time-since-last-seen greeting

CONVERSATION  → Claude API conversation loop

SELFIE_ASK    → Beg for selfie with arm gestures

SELFIE_CAPTURE→ Countdown, capture, show QR

LINKEDIN_FLOW → Search profile, send AI message

GOODBYE       → Wave, happy wiggle, return to IDLE

## LinkedIn Premium Bypass

The "Send me our selfie!" trick creates inbound messages without Premium:

- Johnny captures selfie, shows QR for retrieval

- Johnny sends connection request via lets-connect

- Message includes: conversation recap + "Send me our selfie so I have it too!"

- Person messages first → you can reply without Premium

- If no response → Johnny follows up asking for the photo

## Repositories

- github.com/Utilitron-Robotics/johnny5 — Main robot platform

- github.com/alanchelmickjr/whoami — Face recognition + Gun.js

- github.com/alanchelmickjr/lets-connect — LinkedIn automation

- github.com/liyiteng/AlohaMini — Base hardware

- github.com/Vector-Wangel/XLeRobot — Camera gantry

## Shopping List

- USB speaker (for PulseAudio)

- DisplayPort to HDMI adapter (for 7" touchscreen)

- 5.5x2.5mm barrel connector (if not already wired for Orin)

## Bonus: Hackathon Benchmark Tasks

If time permits, demo the "hard" tasks that are easy with staged environments:

- Puzzle insertion: Red/green shapes on white board → YOLO + VLA pick-place

- Charger plugging: Fixed receptacle, color-coded port, OAK-D depth alignment

- Pouring liquid: Colored cup, fixed pour height, ACT policy from 20 demos

## After Hackathon

- memoRable integration (predictive AI memory)

- Fleet deployment (thousands of Johnnys)

- Domestic tasks (fold laundry, pour drinks, greet visitors)

- XLeRobot.store kit productization

Let's make Johnny Five alive.

— Alan & Opus